Capacitor replacements Atlas 350XL

PC-540 – Pre-driver / SWR protect

| Board | Designator | Original value / type | Recommended replacement |
| --- | --- | --- | --- |
| PC-540 | C501 | 15 µF 16 V Tantalum | 15 µF 25–35 V 105 °C low-ESR electrolytic |
| PC-540 | C505 | 22 µF 16 V Tantalum | 22 µF 25–35 V 105 °C low-ESR electrolytic |
| PC-540 | C506 | 1 µF 50 V Electrolytic | 1 µF 50 V 105 °C electrolytic or 1 µF film |

(Other C50x on this card are 0.01 µF discs or trimmer C509; leave them.)[1]

PC-530 – Driver / bias regulator

From the improved PC-530 block: “C531… 22 mf 25V Electrolytic”.[1]

| Board | Designator | Original value / type | Recommended replacement |
| --- | --- | --- | --- |
| PC-530 | C531 | 22 µF 25 V Electrolytic | 22 µF 35 V 105 °C low-ESR electrolytic |

(Other “C53x” text there are actually resistors or non-caps in the OCR; only C531 is an electrolytic on this card.)[1]

PC-550 – Power amplifier

From the PC-550 CIRCUIT COMPONENTS block in the new OCR:[1]

- C552, C554, C559 – 0.01 µF 100 V disc → keep

- C553, C555, C556, C557, C562, C563 – 330 pF DM-19SM → keep

- C565, C567 – 0.001 µF 100 V disc → keep

- C570 – 100 pF DM-19SM → keep

- “100 mf 16V Electrolytic” (rail decoupler, designator is C561 on schematic) → replace

- C558, C560, C568, C569 – 0.1 µF 100 V Mylar → replace

- C564, C566 – 22 µF 25 V Electrolytic → replace

Updated table:

| Board | Designator | Original value / type | Recommended replacement |
| --- | --- | --- | --- |
| PC-550 | C561 | 100 µF 16 V Electrolytic | 100 µF 25–35 V 105 °C low-ESR electrolytic |
| PC-550 | C564 | 22 µF 25 V Electrolytic | 22 µF 35 V 105 °C low-ESR electrolytic |
| PC-550 | C566 | 22 µF 25 V Electrolytic | 22 µF 35 V 105 °C low-ESR electrolytic |
| PC-550 | C558 | 0.1 µF 100 V Mylar | 100 nF 100 V X7R MLCC or 100 nF 100 V polypropylene film |
| PC-550 | C560 | 0.1 µF 100 V Mylar | 100 nF 100 V X7R MLCC or 100 nF 100 V polypropylene film |
| PC-550 | C568 | 0.1 µF 100 V Mylar | 100 nF 100 V X7R MLCC or 100 nF 100 V polypropylene film |
| PC-550 | C569 | 0.1 µF 100 V Mylar | 100 nF 100 V X7R MLCC or 100 nF 100 V polypropylene film |

PC-250 – 2nd IF / 2nd mixer / mic / S-meter

From the PC-250 CIRCUIT COMPONENTS block in the new OCR:[1]

- C202 – 100 µF 16 V Electrolytic

- C208 – 47 µF 35 V Tantalum

- C220, C221 – 2.2 µF 16 V Tantalum

- C215 – 6.8 µF 10 V Tantalum

- C222 – 22 µF 16 V Tantalum (explicit in the new text, not just “22 mf 16V”)

- C224 – 15 µF 16 V Tantalum

Updated table:

| Board | Designator | Original value / type | Recommended replacement |
| --- | --- | --- | --- |
| PC-250 | C202 | 100 µF 16 V Electrolytic | 100 µF 25–35 V 105 °C low-ESR electrolytic |
| PC-250 | C208 | 47 µF 35 V Tantalum | 47 µF 35–50 V 105 °C low-ESR electrolytic |
| PC-250 | C220 | 2.2 µF 16 V Tantalum | 2.2 µF 25–35 V low-leakage electrolytic |
| PC-250 | C221 | 2.2 µF 16 V Tantalum | 2.2 µF 25–35 V low-leakage electrolytic |
| PC-250 | C215 | 6.8 µF 10 V Tantalum | 6.8 µF 25 V 105 °C electrolytic |
| PC-250 | C222 | 22 µF 16 V Tantalum | 22 µF 25–35 V 105 °C electrolytic |
| PC-250 | C224 | 15 µF 16 V Tantalum | 15 µF 25–35 V 105 °C electrolytic |

(Small discs—0.01, 0.001, 0.1 µF—stay.)[1]

PC-320 – Audio / AGC / oscillator switch

From the PC-320 CIRCUIT COMPONENTS section in the new OCR:[1]

- C301 – 22 µF 50 V Mylar

- C303 – 47 µF 6 V Tantalum

- C305 – 15 µF 16 V Tantalum

- C306 – 1 µF 50 V disc (electrolytic/mylar)

- C311 – 75 µF 15 V Electrolytic

- C313 – 1 µF 25 V Tantalum

- C314 – 2.2 µF 16 V Tantalum

- C315 – 22 µF 16 V Tantalum

- C318 – 100 µF 16 V Electrolytic

- C319 – 0.05 µF 25 V disc (can stay, audio shaping)

- C320 – 250 µF 20 V Electrolytic

Updated table:

| Board | Designator | Original value / type | Recommended replacement |
| --- | --- | --- | --- |
| PC-320 | C301 | 22 µF 50 V Mylar | 22 µF 35–50 V low-leakage electrolytic or new 22 µF film |
| PC-320 | C303 | 47 µF 6 V Tantalum | 47 µF 16–25 V 105 °C electrolytic |
| PC-320 | C305 | 15 µF 16 V Tantalum | 15 µF 25–35 V 105 °C electrolytic |
| PC-320 | C306 | 1 µF 50 V (electrolytic/mylar) | 1 µF 50 V 105 °C electrolytic or 1 µF 63 V film |
| PC-320 | C311 | 75 µF 15 V Electrolytic | 82–100 µF 25 V 105 °C low-ESR electrolytic |
| PC-320 | C313 | 1 µF 25 V Tantalum | 1 µF 35–50 V 105 °C electrolytic or film |
| PC-320 | C314 | 2.2 µF 16 V Tantalum | 2.2 µF 25–35 V 105 °C electrolytic |
| PC-320 | C315 | 22 µF 16 V Tantalum | 22 µF 25–35 V 105 °C electrolytic |
| PC-320 | C318 | 100 µF 16 V Electrolytic | 100 µF 25–35 V 105 °C low-ESR electrolytic |
| PC-320 | C320 | 250 µF 20 V Electrolytic | 220–270 µF 25–35 V 105 °C low-ESR electrolytic |

PC-350 – AF notcher / ANL

From the PC-350 CIRCUIT COMPONENTS block: C353 is clearly 15 µF 16 V tantalum.[1]

| Board | Designator | Original value / type | Recommended replacement |
| --- | --- | --- | --- |
| PC-350 | C353 | 15 µF 16 V Tantalum | 15 µF 25–35 V low-leakage or general-purpose 105 °C electrolytic |

Other C35x are small discs or 0.001 µF polystyrene, so they stay.[1]